| Component | Model selected | Why | Price | Links |
| --- | --- | --- | --- | --- |
| MPU6050 | 6 DOF | - Works on 3.3V Available in Makers - I2C - Has address-select - Well documented | 100L.E | https://makerselectronics.com/product/mpu-6050-imu-sensor-module-with-6-dof-gyroscope-accelerometer https://components101.com/sites/default/files/component_datasheet/MPU6050-DataSheet.pdf |
| BNO055 | GY-BNO055 Generic Modules | Way cheaper than other models Runs natively on 3.3V Has internal pull-up resistor Con: Poorly documented | 1.5k L.E | https://www.ubuy.com.eg/en/productuk/TMOZQIX5W-electronic-component-sensor?srsltid=AfmBOoqqVjcFuJ_IQJfS3Z1sBZmRBwacuiCxhf3_XhbrO8dlaIZxF1lM Good reference: https://forum.arduino.cc/t/gy-bno055-issues/551978/19 Good for conn (Mentions STM32): https://sigmanortec.ro/en/absolute-orientation-sensor-module-9-axes-gy-bno055-iic-serial-3-5v |
| Ultrasonic | HC-SR04 | - Cheap - Low current draw (15mA) - Imp: Op voltage is 5V so will need voltage div | 55 L.E | https://store.fut-electronics.com/products/ultrasonic-sensor-module?srsltid=AfmBOoq_YJXfbLI1iDC6IVOKMJdZhO4iG-qxw_L6wWBZfpXa_Jaf7edE https://components101.com/sensors/ultrasonic-sensor-working-pinout-datasheet https://components101.com/sites/default/files/component_datasheet/HCSR04%20Datasheet.pdf |
| Limit switch |  |  |  |  |
| ESD Protection | USBLC6-2SC6 | Protects two differential data lines and VBUS simultaneously (protects both SDA and SCL in a single 6-pin package) Well documented | 12L.E | https://uge-one.com/product/usblc6-2sc6-esd-protection-tvs-unidirectional-dual-array-sot23-6/?srsltid=AfmBOoqaDx91zqxQXQ-GpplYuqR-loDjQQTuFoIu7uGVFHpXsEGgcTto https://www.st.com/resource/en/datasheet/usblc6-2.pdf |

# Notes or Tip & Tricks:

- Operating V: Check if it runs on 3.3V or higher. If for example the component outputs 5V on the STM32’s 3.3V GPIO pin, we need a Voltage div

- Current draw: How much current does the component pull & can our PDB supply it along with the other stuff

- Communication protocol: I2C, SPI, UART, or analog/digital GPIO

- If I2C, does the address collide w/ anything else alr on the bus? (Check from I2C address datasheet).

- In the datasheet, we gotta consider some critical things:

- Pinout diagram

- Max ratings table (Max V, Max I, Max T) -> never exceed

- Normal op range (typical V, typical I draw, op T range)

- I2C/comm address section (for anything digital-bus based) ->Default addresses & how an address-select pin changes it (if available)

- Typical application or circuit diagram -> reference schematic

- Layout recs/PCB guideline

- Package/mechanical dimensions